Проект с камерой на мега 162, BASCOM AVR

Вот проектик, который в очередной раз доказывает превосходство контроллеров AVR над остальными процессорами.

При включении устройства, инициализируется экран, заливается черным цветом и выводится видео с камеры в реальном времени.

Железо:

Камера  ADCM1700 берется из телефона Siemens С65 (их там несколько видов надо брать с надписью HERA 0408)

Контроллер ATMEGA162 – 16PU. Можно гнать до 24 мегагерц (корпус кварца цепляем на массу, так стабильнее работает).

Дисплей от Siemens S65 c контроллером LPH8836 (другие дисплеи с другими контроллерами ставить нет смысла ввиду их разных команд инициализации)

MEGA тактирует камеру выводом OC1A частотой 8 мегагерц. Также с камеры приходят сигналы синхронизации hsync и vsync, которые подцеплены на прерывания контроллера.

Дисплей висит на шине SPI, которая разогнана до предела, и в основном влияет  на всю производительность устройства.

Питание устройства 3 volt.

Питание подсветки дисплея 12 volt.

Project with camera on mega 162, BASCOM AVR

Here is little project, which in the next once proves the superiority a controller AVR on rest processor.

When enabling device, is initialized screen, is flooded by black colour and is removed video real-time with cameras.

The Iron:

The Camera ADCM1700 rs from telephone Siemens S65 (their there several types it is necessary to take with inscription HERA 0408) Controller ATMEGA162 - 16PU. Possible drive before 24 megahertz (the body of the quartz fastened on mass, so more works).

The Display from Siemens S65 with controller LPH8836 (other display with the other controller to put no sense in view of their different commands to initialization) MEGA clock the camera by conclusion OC1A frequency 8 megahertz. Also with cameras come the signals to synchronizing hsync and vsync, which are hooked on  interruptions of the controller.

The Display hungs on bus SPI, which is dispersed before limit, and basically influences upon the whole capacity device.

Feeding of device 3 volt.

Feeding of highlight display 12 volt.