REMOTE CIRCUIT OPERATION VIA PHONE

This describes how a remote circuit can be activated using the phone and without logging a call. This is a method that can be used for non-critical applications.

The activation involves a person ringing the number of the remote site in a way that the remote circuit decoder can interpret and act on. This decoder module is placed in close proximity to a phone and relies on the audio produced from the phone ring.

For an action to occur, the initiator must let the phone ring for a required number e.g. 4 rings. One minute later, another call is made and the number of rings determines which action occurs, e.g. 2 to turn on circuit 1, 3 for circuit 1 off etc.

There are a lot of limitations to this communication system but it can be useful for many non-critical applications such as turning on a sprinkler system, operating lights and many other possibilities.

Some limitations on this method are that there is no feedback as to whether the wanted action occurred. There may also be a nearby loud noise at the remote end that disrupts the correct action interpretation.  A more certain method can be to tap the phone lines but an electrical connection in most cases involves equipment approval from the regulatory body.

The system described above uses a combination of analog circuitry and digital with an ATMEGA8535 processor.

# Circuit description

The TLV271 op amp whose gain is set by two slide switches amplifies the microphone electrical output in the decoder module. The closer the module to the phone, the less gain is needed and thus less susceptibility to extraneous noise from external sources. The output from the op amp feeds the LM556, which is arranged as a one shot  multivibrator. The output of the LM556 is fed to an LED for diagnostic purposes as well as a counter input and an input pin of the MEGA8535. There is also a LCD module, which is useful for diagnostic purposes when analysing the decoder processing. Outputs from the MEGA8535 feed relay circuits whose contacts are brought out via a socket. The relays are arranged so that they don’t draw any current from the regulated 5 volts as they connect directly to the input 12V supply, which is from a plug pack.

The decoder is housed in a small plastic box approx 150 x 90 x 45mm with the LCD at the top and the microphone and LED at the front.

## Possible external circuit configurations

Apart from just using the contact outputs to activate circuits directly, there are cheap commercially available radio systems where remote power points can be turned on and off by pressing buttons on the transmitter. It is feasible that the floating relay contacts could be used to replace these buttons by modifying the transmit module.