Fiber-optic Christmas tree

I wanted to joy myself and my close people on New Year’s and I bought a synthetic Christmas tree with an FO illumination IGT040EG1. The tree was 40 cm high and was powered by 3 AA batteries with total voltage 4,5 V. I switched the tree on in a dark room and prepared to see a picturesque light show, but I was surprised when the tree began to flash red and yellow in turn. This synthetic wonder was operated by a small board with a bun-like chip, which switched two bright LEDs of red and yellow color in turn. Among the tree branches there was a separate hanging garland made of LEDs of white color, which shone brightly by switching on the tree. At the top of the tree was a transparent plastic star with some LEDs inside. The star was electrically connected with the garland and was switched on with it at the same time. This is strictly speaking all that I can say about this original product of Chinese industry.

I supposed that technical possibilities of the tree allow to reproduce more lighting effects and I have worked out a control board on microcontroller Altmega 8L-8, which is mounted instead of the old one. In this new variant the tree creates very beautiful light patterns and dynamic effects, it can work in 4 modes: automatic, acoustic clap switch, acoustic-interactive, photodetector. Here are some of the realized lighting effects: fast switchings of three-colored LEDs (for each color), lighting effect «shimmer» (for each color), color mixing, fast switching on – fluent switching off, consecutive switching of LEDs crystals with an effect of light accumulation and changing, color impulses with a small pause (for each color), increase of flash frequency, alternate switching from tree to garland with glow color changing, etc. The use of bright RGB-LEDs allows to watch lighting effects by the daylight, but the best results turn out in a dark room or by the switched off overhead illumination.

The garland situated on the tree shone too bright, so I had to limit programmatically the glow brightness level. For energy economy and for luminous emittance leveling of the garland and the tree the garland operating occurs with the impulse duration less than 50% from the initial. Besides, the garland is switched on not in all program units but only in the last three. The garland reproduces the following lighting effects: flashes, fast switching on – fluent switching off, switchings, shimmering.

Switching from one mode to another is realized cyclic by the short pressure of the button on the upper frame cover. The two-colored LED (red-blue) placed near fulfils service options indicating active working mode.  An external interrupt at the microcontroller INT0  output is used for button SB1 operating. That’s why button mode switching can be made at any moment.

In the automatic mode the indicating LED is not shining, there is no need to push the mode choice button. The automatic mode starts just after the power is switched on (at the frame bottom) and then reproduces cyclic programmed lighting effects. The mode changing is possible by single short pressures of the button SB1. Control board reproduces cyclic 12 complicated program units, which create an original light pattern on the tree. Every lighting effect is reproduced for each color, so the total amount is 36 effects. A single effect for every color is repeated many times, it also increases the total amount of reproduced effects. The automatic mode is convenient because it doesn’t require efficient intervention; the program makes everything creating different light patterns on the tree surface.

In the acoustic mode «сlap switch» the indicating LED HL1 shines continuously with red color. The mode is switched on by one short pressure of the button SB1. The tree switches on only after the third clap and it will continue to work autonomously reproducing lighting effects. At the clap moment the indicating LED is going out for a split second, it confirms the received sound signal. After the third clap all following claps won’t be effective. For restart the button SB1 should be pushed 3 times. This mode can be used at parties and holidays for attracting attention and creating a mysterious atmosphere.

Switching on the second acoustic mode «interactive»: the button SB1 should be pushed 2 times (if at the beginning) or once if the previous mode is changing. The indicating LED will shine blue, and then it’s possible to start interactive three-colored composite effect. The tree will respond to every sound signal with fast successive reproducing of three colors: blue, red and green. The sensitiveness to sound signals in this mode is very high and the device will react even to human speech. The color symphony can play a role of an accompaniment to karaoke songs or to music.

Switching on the mode «photodetector»: the button SB1 should be pushed 3 times (if at the beginning) or once if previous mode is changing. When the mode is switched on the indicating LED HL1 will flash red. It means that the beginning of a programmed test of the room illumination level. That’s why it’s recommended to switch on this mode by the already turned on general illumination of the room. The slightest darkening or turning the light off will lead to reproducing programmed lighting effects. The specific of the mode «photodetector» is that just the light in the room is turned on once again the lighting effect on the tree switches off. The following turning-off the light in the room lights the illumination on the tree again etc… The reason is that the phototransistor is connected to the second microcontroller external interrupt INT1, which allows to start or to stop efficiently the lighting effects program.

The next pressure of the button SB1 switches the program into the first mode. In order to prevent the bounce the stoppings in the interrupts are a bit more than usual. It shows by sight as a rather late mode switching.

The electric scheme is based on the microcontroller Atmega8L-8 with a minimal supply voltage level ca. 2,7V. The choice of this microcontroller is caused by the limit of supply voltage in form of a tree batteries set with common voltage 4,5V. To prolong the batteries operating life and to raise efficiency all component parts are powered by a pulse-duration conversion device MC34063AP. Out voltage 3,5V after resistors R6R8 comes to the microcontroller, indicating LED HL1, three-colored LEDs HL2-HL5, note amplifier, and transistor VT1.

For sound signals reception and analysis I’ve built a low-frequency amplifier on transistor VT1. Sound signal appears on the microphone port BM1 as weak electric impulses. They come through the capacitor C3 and the transistor amplifier’s base where they have voltage amplification. Then the intensified sound signal comes from the upper (due the scheme) resistor terminal R5 to the microcontroller’s port 28 ADC5 and is digitalized. Then the volume level and the pause between signals are programmatically analyzed. For the second mode «clap switching » the acoustic signal becomes considerably weaker and the pause control between signals is used.  On the contrary, for the mode «acoustic – interactive» the signal amplification is programmatically realized and the tree can react to common human speech. The device sensibility to sound signal can be changed not only programmatically but also by changing the resistance of the resistor R5.

The following is programmed in the microcontroller DD1 cell:  the routines of lighting effects reproducing and the signals of out sensors of phototransistor VT2, button SB1 and amplifier output. The inner generator RC 4 MHz is hooked up programmatically. The program is written on BASIC in BASCOM-AVR software-envelope and is a bit more than 4Kb. To set fuse bits by programming is not needed. Resistor R16 disables an inadvertent trip-out by voltage reduction in the feed network.

Crystal operating of each HL2-HL5 LEDs is organized due the program.  In order to create a great variety of effects all cathodes are hooked up through the FET VT1, which functions in pulse-time modulation mode. This signal comes from the OC1A port and allows turning all three-colored LEDs on and off fluent. The garland on the tree is operated by the same way. It’s operated by the global bus which is hooked up through the FET VT4. The operating signal comes from the microcontroller to FET VT4.  The impulse duration in the signal is changed due the given program. It shows in different lighting effects on the garland. The microcontroller not only operates RGB-LEDs but also combines their switching on with the LED-garland. As a result there are composite lighting effects.

Parts and materials

The printed-circuit board (dr.2) is made from a single-sided glass-fiber laminate sheet with thickness 1,5-2 mm. The dimensions are 60,2x60,2 mm. Radio elements are mounted due the dr. 3. The list of radio elements you can see in the table1. The shape of the printed-circuit board has a figured notch where plastic pin for a screw should be placed after the installation of the board in the frame.

Please pay attention to phototransistor VT2, two-colored LED HL1 and button SB1 setting. They are mounted after all radio parts. You need to prepare 2 holes for phototransistor and LED on the upper frame cover beforehand. By the mounting of phototransistor and LED the length of legs should be so that the upper cover can close free and the frames of the details slightly outlook from the holes.

After marking-off  you need to make two holes on the upper cover for the button setting (see the photo). The diameter of the holes is ca. 1 mm. The button pins are passed through the holes and are incurved from the opposite side. The button is connected with the board with 2 bounding wires. There’s no need to make special holes for the microphone, the high sensibility of the scheme allows receiving sound signals in close frame.

Here can be used any 3 mm two-colored LED of red and blue colors.

List of radio parts

| Position | Name | Comment | Quantity, pcs |
| --- | --- | --- | --- |
| DD1 | Altmega8L-8 | Replacement is not recommended! | 1 |
| DA1 | MC34063AP |  | 1 |
| VT1 | BC547 |  | 1 |
| VT2 | ФТ-2К | phototransistor | 1 |
| VT3,VT4 | КП505А | BS170P | 2 |
| R1 | 5,1 kO | 0,125W | 1 |
| R2 | 470kO | 0,125W | 1 |
| R3 | 22kO | 0,125W | 1 |
| R4 | 1 kO | 0,125W | 1 |
| R5 | 120 kO | 0,125W | 1 |
| R6 | 1,2 kO | 0,125W | 1 |
| R7 | 1,5 mO | 0,125W | 1 |
| R8 | 2,2 kO | 0,125W | 1 |
| R9-R11,R13-R15,R17-R22 | 220 О | 0,125W | 12 |
| R12 | 470 О | 0,125W | 1 |
| R16 | 2 kO | 0,125W | 1 |
| BM1 | DN9767 | Electret small microphone | 1 |
| C1,C6 | 220uF10V |  | 2 |
| C2 | 22uF6,3V |  | 1 |
| C3,C5,C7,C8 | 0,1uF |  | 4 |
| C4 | 470picofarad |  | 1 |
| L1 | 240uH |  | 1 |
| VD1 | 1N5819 |  | 1 |
| HL1 | L57EGW or analogue |  | 1 |
| HL2-HL5 | L5013RGB |  | 4 |
| SB1 | Two-pin- button |  | 1 |
| XT1 | Two-sectional - joint |  | 1 |
| - | Panel for microcontroller | DIP28pin | 1 |