Ignition angle measuring device for two stroke internal combustion engine

This measuring device was made for testing electronic ignition control for two stroke engines in real time. The electronic circuit of it is the result of a minimal modification of a circuit designed for ignition control. I think it can be further simplified regarding the external clock working at 200KHz and providing sign for port P1.0 as well as the size of microcontroller (AT89s52), but this was not the aim of the project.

The device receives - as inputs - interrupt at the upper dead point of the revolving crankshaft and at the point of ignition.  Between these interrupts the device measures, by polling, the time, and, by using serial communication, it sends the data to a computer for further processing. I used serial/USB converter for a laptop not having serial communication port and it worked well with it.

I tested my ignition control program with this measuring device using the sweep function of a waveform generator starting from 3Hz (180 rev/minute) and running as far as 100Hz (6000 revolution/minute) during 500 sec running time. A real testing during driving a car can also be done as well.

The corresponding program of the computer (not provided) receives the time data in real time in form of synchronized dual packages and places it alongside the corresponding angular speed of the crankshaft in an excel table (for example at right of the reference column to the spectrum of angular speeds - in my case having an interval or step of 10 rev/minute). Overwriting happens. The ignition angle (max and min value) to a certain angular speed of the crankshaft then can easily be calculated using the time data and taking into account the number of engine cylinders during the calculation.

Mr. Zsolt FAZEKAS (fazekaszs@hotmail.com)